|  | Appendix 2a Statement of Requirements – Pumping Appliances NFCC Emergency Response Vehicle Framework DS478-24 |
| --- | --- |

2026 Devon and Somerset Fire and Rescue Service

All rights reserved.

No copying, selling, public performance, hire

or distribution in any form without the permission of the

Chief Fire Officer.

Contents

Section One – Instructions and Supporting Documents	3

Section Two – Introduction to Requirements	5

Section Three – Vehicle Requirements	6

1.	Chassis / Base Vehicle	6

2.	Power Take Off (PTO)	9

3.	Cab Exterior	11

4.	Crew Cab Interior	13

5.	Crew Cab – Number of Seats / Fire Crew	15

6. 	Water Installation and Tank	17

7. 	Pump, Controls and Instrumentation	19

8. 	Foam / CAFS	21

9. 	Hose-reel	23

10. 	Roof Walkways	25

11. 	Vehicle Markings & Livery	27

12. 	Bodywork & Body Stowage	29

13. 	Stowage Lockers	31

14.   Stowage Tunnel	33

15. 	Alternative Power Generating Systems	34

16. 	Electrics	36

17. 	Vehicle Battery and Charging	38

18. 	Appliance Lighting	40

19. 	Scene Lighting	42

20. 	Telescopic Mast Light	44

21. 	Ladder Stowage and Gantries	45

22. 	Vehicle Inventory	47

23. 	Crew Cab Stowage	49

24. 	Communications & ICT	50

25. 	Radio Installation	52

26. 	CCTV	54

27. 	Vehicle Control Systems	55

28. 	Emergency Visual and Audible Warning Equipment	57

29. 	Additional Requirements	59

Section Four – Equipment List	60

# Section One – Instructions and Supporting Documents

Instructions

For each section of the vehicle requirements, it is split out into three sub-sections as follows:

- Compliant framework requirements – highlighted in grey

- Contracting Authority’s requirements

- Section questionnaire

The Statement of Requirements (SORs) have been laid out as follows to ensure a standard approach is taken in a format most suitable for both Parties.

Suppliers

The Suppliers must:

- confirm they comply with the framework requirements by ticking yes to the declaration

- submit a response to Contracting Authority’s requirements by adding yes into the compliant column and add a detailed response into the Supplier’s response column to show how the requirement will be met.

- submit a response to the Section questions in the response box for each question.

The Supplier must submit supporting documentation or evidence to support their response. Any attachments and supporting documents must be clearly marked with the reference number of the relevant question(s).

Please refer to the assessment matrix for details on how each element will be scored and for a summary of the questions included in the Invitation to Tender.

Contracting Authority / FRSs

Contracting Authorities should review the requirement and available information on the Framework and expand or clarify information if required. Please ensure that you reference any appendices within your requirements.

Contracting Authorities are required to fill out all sections within this document.

Contracting Authorities can remove or amend the questions in the sections below.

Please ensure that you delete any guidance notes before submitting your ITT to the Suppliers.

Please note that Appendix 2a – Standards and Legislation may not capture all standards and legislation applicable to your procurement or those captured may have been superseded. Contracting Authority/FRSs should review and update as necessary for their procurement.

Section Questions

[Contracting Authorities may want to consider word limits]

Suppliers should read the following guidance prior to completing the section questions.

- All suppliers must provide a response to each question.

- The questions are split into two different categories:

- Pass/Fail - A Supplier must pass all pass/fail questions for the vehicles types they are submitting a response for. The pass/fail marking guidelines is provided below. A fail against any assessment question for a particular vehicle type will constitute a fail for that vehicle type;

- Scored – The Suppliers responses to these questions will be scored 0-4, using the marking guidelines provided below. A score of 0 against any question will constitute a fail. A fail against any assessment question for a particular vehicle type will constitute a fail for that vehicle type. Questions may be weighted.

- To be successful with this element of the assessment, Suppliers must:

- Pass all pass/fail questions;

- Score 1 – 5 against all scored questions (a 0 against any constitutes a fail).

- The marking guidelines provided with each question are provided as a guide for what is expected to be included within the Suppliers response. Suppliers should also consider the requirements when writing their response.

Scoring of responses shall be as defined in the marking guidelines below:

Marking Guidelines for the Pass/Fail Questions

The pass/fail questions shall be scored against the following marking guidelines:

| Marking Scheme | Description |
| --- | --- |
| Pass | The response provided meets the requirement. |
| Fail | The response provided does not meet the requirement. For the avoidance of doubt, a 'fail' against any mandatory pass/fail question will result in a failed tender submission and exclusion from the procurement process. |

Marking Guidelines for the Scored Questions

The scored questions shall be scored against the following marking guidelines:

| Marking Scheme | Description |
| --- | --- |
| 5 | Comprehensive, unambiguous and useful response which demonstrates a thorough understanding of the requirements and provides details of how the requirements will be met in full. |
| 4 | Response satisfies the requirements and includes a good level of detail to meet the basic requirements. |
| 3 | Response meets minimum requirements but remains basic and needed to be expanded upon. Sufficient but does not inspire. Limited evidence to support response and/or one a deficiency apparent. |
| 2 | Response which only partially satisfies the requirement with apparent deficiencies and not all elements of the requirement being delivered posing a risk that the requirements will not be met. Falls short of minimum expectations and may lack supporting evidence to demonstrate the specification can be met. |
| 1 | Poor response, only attempted to answer the requirement, major deficiencies. Insufficient or no evidence to support the response. An unacceptable response with serious reservations, demonstrating no understanding of the requirement. |
| 0 | The response does not meet any of the requirements or no response has been provided. A score of 0 against any questions shall constitute a fail. For the avoidance of doubt, a 'fail' against any mandatory pass/fail question will result in a failed bid and exclusion from the Framework. |

Supporting Documents

Note that this document is only one document that forms the Further Competition, the following documents are included as part of the Further Competition.

| Invitation to Tender |  |
| --- | --- |
| Appendix 1a | Assessment Matrix |
| Appendix 2a | Contracting Authority Statement of Requirements for Pumping Appliances |
| Appendix 2b | Standards and Legislation Requirements |
| Appendix 2c | Contracting Authority Statement of Requirements for Non-Technical Requirements |
| Appendix 3 | Call-Off Terms |
| Appendix 4 | Form of Tender and Declarations |
| Appendix 5 | Pricing Schedule |
| Appendix 6 | DRAFT Call-Off Order Form |

The Framework has set out Technical (Appendix 2a - Statement of Requirements for Pumping Appliances) and Non-Technical requirements (Appendix 2c - Statement of Requirements – Non-Technical) and the Standards and Legislation (Appendix 2b – Standards and Legislation) that must be adhered to.

# Section Two – Introduction to Requirements

The following information provides an overview of the Contracting Authority/FRSs requirements.

| Name of Contracting Authority/FRS |  |
| --- | --- |
| Internal Project / Procurement Reference |  |
| Summary of your FRS and requirements |  |
| Number of Vehicles |  |
| Delivery Address |  |
| Expected Life for the Vehicle(s) |  |

# Section Three – Vehicle Requirements

Stage One – Chassis

## 1.	Chassis / Base Vehicle

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | The vehicle shall be configured for right-hand driving and conform in all aspects to the requirement of EN 1846 unless specified by the Contracting Authority. |
| --- | --- |
|  | The driving position shall be capable of being optimised to facilitate maximum all-round vision and therefore achieve the best possible visibility for a diverse range of drivers. No objects, equipment or labelling must be placed or located on the dash or windows which will affect the vision of the driver and comes under the categorisation of defects February 2021 v1.0 DVSA categorisation of defects. |
|  | The Supplier must ensure that an additional form of retardation to normal braking e.g., a retarder and a method of manually operating the retarder. Any such retarding system shall not have a detrimental effect on the handling of the vehicle. |
|  | Enhanced traction control and other electronic safety technologies are required by the Contracting Authority to meet its operational needs. |
|  | Vehicles fitted with fixed anchorage points (towing eyes or balls) at the front and rear of the vehicle as required by BS EN 1846 shall be tested, certified, and marked with Safe Working Load. in KG. Towing eyes must be marked for use, which must be agreed with the Contracting Authority. Anchorage points for working at height, on the Appliance roof shall be sufficient to enable their use as anchorage points for restraint/fall restraint purposes), and likewise tested, certified, and marked with SWL. |
|  | Contracting Authorities may have a requirement for a vehicle mounted winch system of a capability determined through operational risk assessment. Where a vehicle-mounted winch is stipulated, it shall be fitted in such a manner as to minimise the hazard presented to other road users and must not affect any vehicle safety systems e.g., advanced driving systems, radar etc. |
|  | The vehicle must be fitted with specified tyres with a specified load rating and speed rating. These must be suitable for the full range of blue light emergency operational use reasonably anticipated by the Contracting Authority for the vehicle and be demonstrated as such through approval testing and lifetime considerations such as steering, braking, maximum axle loadings and wear rates. |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |
|  | The vehicle transmission shall be automatic / semi-automatic / manual. |  |  |
|  | The vehicle tyres shall be: Type of Tyres – [add in] Load rating - [add in] Speed rating - [add in] |  |  |

The following table shows the questions that relate to the Chassis / Base Vehicle section:

[please note question T-1a is an example for the template purposes]

| Ref | Question | Weighting |
| --- | --- | --- |
|  | Question Topic – Chassis Question: Please provide details of the proposed chassis and engine and provide technical documentation from the chassis manufacturer to support your response. Your response shall include, but not limited to: • A description on the proposed chassis and engine selected • The chassis considered, the reasons for selecting the chassis detailed within your response and how did you make the chassis decision • Technical documents from the chassis | Pass / Fail |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 2.	Power Take Off (PTO)

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | The pump drive shaft to be as straight as possible, to reduce noise & mounted within the tolerances specified by the shaft / universal joint manufacturer. |
| --- | --- |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the PTO section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 3.	Cab Exterior

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | The Contracting Authority requires the vehicle to be fully lockable (including any equipment lockers) operated from the driver’s position and any other positions required by the Contracting Authority to support operational requirements as determined through appropriate risk assessment. |
| --- | --- |
|  | The Contracting Authority may require civil disturbance protection systems, such as (but not limited to) reinforced glass or specialist cab door locking systems. |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Cab Exterior section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 4.	Crew Cab Interior

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | The design of the vehicle shall maximise the safety of operators and passengers through the provision of occupant protection systems. All ancillary installation (such as Mobile Data Terminals (MDTs) etc.) shall be fitted so as to not impede the operation of airbags and other safety devices fitted. Cab and crew cab crash protection shall be afforded by certification to ECE 29 a, b and c. |
| --- | --- |
|  | Unimpeded access and egress of all doorways and other access points will be provided for all crew/personnel wearing different configurations of Personal Protective Equipment and Respiratory Protective Equipment. High visibility grab handles shall be fitted at all access points to allow three points of contact and appropriate lighting to illuminate the steps and area around the doors. |
|  | The Contracting Authority may require the Vehicle/s to be fitted with equipment to enable remote monitoring from within the vehicle cab. For example (but not limited to), reversing sensors, cameras, proximity devices, CCTV and telematics data gathering. Such devices may be required for safety and/or evidence gathering. Please detail CCTV requirements in section 25. |
|  | The Supplier shall ensure the Vehicle provides adequate heating and ventilation to maintain crew comfort and glass demist. |
|  | Noise levels within the crew cab shall comply with the requirements of The Control of Noise at Work Regulations 2005 and shall be less than 80dB(A). |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Crew Cab Interior section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 5.	Crew Cab – Number of Seats / Fire Crew

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | The seatbelts fitted to the vehicle shall allow for a reasonable degree of free movement that will be facilitated through the provision of three-point inertia adjustable seat belts. All crew seat belts, and their anchorage points shall provide the same level of occupant restraints and shall conform to relevant regulations. |
| --- | --- |
|  | All seats shall be fitted with occupancy warning devices with audible and visual warnings displayed to the driver to indicate seat occupancy and seat belts use. |
|  | The seatbelts must be a bright colour e.g., yellow or red, to make them easily differentiated from the BA straps in ‘rig on route’ seating. |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |
|  | The occupancy of the vehicle is [number]. |  |  |

The following table shows the questions that relate to the Crew Cab – Number of Seats / Fire Crew section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

Stage Two – Fire Engineering

## 6. 	Water Installation and Tank

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | The water installation shall be compatible with the firefighting media requirements as outlined in the National Operational Guidance Programme for UK Fire and Rescue Service, both when operated in isolation and when operated in series/relay at a maximum performance of the combined vehicles. |
| --- | --- |
|  | All elements of the water installation, supplies and deliveries shall be able to be isolated individually. |
|  | Water installation output shall be sufficient for operational firefighting. |
|  | All ‘low pressure’ inlets and outlet hose coupling shall be compatible with BS 336 instantaneous couplings. Please see 8.2 for CAFS coupling requirements. |
|  | The water installation design must fully consider the engine and power take off torque requirements of the vehicle so as to optimise the engines power and efficiency. The resultant noise levels must not exceed the first level of protection. |
|  | The water installation shall be designed to be operated with sea water, foul, and contaminated water (possibly containing foreign bodies), mains water, fresh water, and the range of commercially available fire-fighting foam solutions. Anodes or other means to reduce internal corrosion of the pump must be part of the pump design. |
|  | The water tanks must be baffled. |
|  | On board water supply tank(s) shall provide for a continuous delivery to the main fire pump in accordance with the Contracting Authority’s operational requirements and the limitations of Vehicle. The supply shall not restrict the designed performance of the pump. |
|  | The water installation shall be capable of maintaining a continuous supply across the full range of pump performance whilst being augmented from a pressurised source. |
|  | The accurate content of water and firefighting media tanks shall be visually and/or audibly indicated at the pump operator’s position at all times. The level of accuracy of this indication shall be specified by the Contracting Authority. |
|  | Water systems shall be designed to prevent frost damage and facilitate repair and preventative maintenance. |
|  | The Contracting Authority may require delivery of water through an additional fixed or semi-fixed e.g., roof mounted monitors. |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Water Installation and Tank section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 7. 	Pump, Controls and Instrumentation

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | The fire-fighting water pump and its installation shall be capable of receiving pressure fed supplies from a wide and varied range of sources in line with BS 336-2010. The design shall enable these to supply, both directly to the pump from between one and four delivery hoses and to the on-board water supply/tank from between one and two delivery hoses. |
| --- | --- |
|  | Controls that provide variable and finite command of pump performance shall be provided and be capable of being operated with a gloved hand. |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |
|  | A set of pump operating controls within the vehicle cab to engage or disengage and monitor pump operation. |  |  |

The following table shows the questions that relate to the Pump, Controls and Instrumentation section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 8. 	Foam / CAFS

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | Systems for storing and delivering firefighting media other than water will be required as specified by the Contracting Authority. |
| --- | --- |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |
|  | For CAFS, the Contracting Authority may require low pressure outlets with Storz connection. This will allow the Contracting Authority to ensure operators only use the dedicated CAFS outlets when in use. Please see 6.4 for instantaneous couplings requirements. |  |  |

The following table shows the questions that relate to the Foam / CAFS section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 9. 	Hose-reel

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | High-pressure hose reels shall be equipped with a power assisted rewind system. A backup manual rewind system shall be provided. The system shall also include provisions to prevent hose entanglement and enable acute angles of hose deployment. The water delivery performance of the hose reel shall be to the specified performance of the Contracting Authority and meet BS EN 1947:2014. |
| --- | --- |
|  | To ensure that suction and delivery hose can connect to existing water systems, suction and delivery equipment shall provide compatibility from the range of Contracting Authority existing systems as specified. |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Hose-reel section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

Stage Three – Body build

## 10. 	Roof Walkways

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Roof Walkways section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 11. 	Vehicle Markings & Livery

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | Vehicle conspicuity shall comply with the current road vehicle lighting regulations and provide high levels of visibility in all weather conditions and times of day. Should a national identity be implemented during the life of the framework this will form part of the requirement. |
| --- | --- |
|  | The Contracting Authority colour finish of vehicles and bodies will be utilising standard manufacturers colours as close to RAL 3002 Red. |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Vehicle Markings & Livery section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 12. 	Bodywork & Body Stowage

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | The vehicle specification and design shall give due consideration of the chassis manufacturers and industry best practice guidance on overall Vehicle and axle loadings and shall not exceed 90% permissible axle weights and maximum authorised mass of the vehicle, without the expressed permission of the Contracting Authority. Vehicle design, stowage layout shall enable even side-to-side weight distribution with the lowest centre of gravity and shall prevent oversteering. |
| --- | --- |
|  | Road and off road (if applicable) handling characteristics shall be suitable for the vehicles safe use in a range of environments and blue light driving conditions specified by the Contracting Authority. |
|  | Vehicle design shall take account of rigours of operational use and emergency response, driving; for example, through and over traffic calming measures. |
|  | Interlock systems shall be incorporated to enable ancillary systems to operate only whilst the vehicle is stationary, and the parking brake is applied (except where Contracting Authority explicitly specifies otherwise). |
|  | Vehicle dimensions shall be in compliance with the Construction and Use Regulations and BS EN 1846. The Contracting Authority may wish to specify particular dimensions for operational needs, e.g., departure angles or maximum vehicle heights. |
|  | Ultra-high pressure systems may be fitted as specified by the Contracting Authority. |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Bodywork & Body Stowage section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 13. 	Stowage Lockers

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | Vehicles shall provide adequate stowage for an inventory of equipment stipulated by the Contracting Authority, to support operational requirements as determined through appropriate risk assessment. Please see Section Four. |
| --- | --- |
|  | Access to equipment shall be fully and readily adjustable and adaptable to enable reconfiguration of the area as operational needs change throughout the life of the vehicle. Use of the available stowage volume shall be maximised. |
|  | The disposition, security and accessibility of each item of inventory shall be suitable for safe use by a diverse workforce and in accordance with individual Contracting Authority employment policies. Accessibility of each inventory item and manual handling requirements shall be subject to the risk management processes set out in The Manual Handling Regulations for each stowage compartment. A complete copy of the risk assessment is to be provided with the first vehicle at delivery. |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Stowage Lockers section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 14.  Stowage Tunnel

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Stowage Tunnel section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 15. 	Alternative Power Generating Systems

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | The power generation capability shall exceed that required to power all vehicle systems and include a battery guard system. |
| --- | --- |
|  | Auxiliary power requirements may be required to exceed the provisions of BS EN-1846 to support operational requirements as determined by a Contracting Authority through risk assessment. |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Alternative Power Generating Systems section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 16. 	Electrics

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | Electrical installations with removable elements, such as auxiliary equipment shall be able to be isolated and/or insulated from the power supply. Examples of such equipment would be chargers, water heaters, CCTV modules etc. |
| --- | --- |
|  | Installations and their associated equipment shall provide compatibility with the range of existing systems stated by the Contracting Authority |
|  | All wiring to be sheathed and run-in conduit when it’s routed through lockers with no or minimum joins. Where wiring runs through the body work grommets must be fitted, and where applicable wiring must be clearly identified to denote its purpose. Where applicable, wiring/electrical circuits should be suppressed so as not to interfere with communication systems. All fuses, relays, circuit boards, diagnostic points must be easily accessible and clearly marked. |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Electrics section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 17. 	Vehicle Battery and Charging

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | Battery charging, whilst the vehicle is stationary in an appliance bay, shall be provided to support the requirements of a range of on-board systems. For an element of future proofing, the system shall provide a surplus capacity of a level to be agreed with the Contracting Authority. Alternative methods of supplementary charging such as solar should be considered. |
| --- | --- |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Vehicle Battery and Charging section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 18. 	Appliance Lighting

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | All work and storage areas of the Vehicle, including within the cab, shall be equipped with lighting to enable safe operations in low light conditions. |
| --- | --- |
|  | A means of illumination may be required for the Officer in Charge to assist remote identification of street names or house numbers as define by the Contracting Authority. |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Appliance Lighting section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 19. 	Scene Lighting

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | Vehicles may feature elevated directional and/or scene lighting to illuminate scenes to assist safe operations up to a reasonable distance as stated by the Contracting Authority. |
| --- | --- |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Scene Lighting section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 20. 	Telescopic Mast Light

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Telescopic Mast Light section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 21. 	Ladder Stowage and Gantries

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | Provision shall be made for the secure stowage and carriage of ladders, gantries, stowage boxes and other equipment externally on the vehicle during transit. Equipment shall be determined by the operational requirements of the Contracting Authority. |
| --- | --- |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Ladder Stowage and Gantries section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 22. 	Vehicle Inventory

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | The Contracting Authority may require the Vehicle to provide compressed air power via a number of outlets around the Vehicle that meet the requirements to operate pneumatic equipment items within the inventory. An auxiliary air tank may be required. |
| --- | --- |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Vehicle Inventory section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 23. 	Crew Cab Stowage

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Crew Cab Stowage section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 24. 	Communications & ICT

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | Vehicles shall have provision for the communication and information technology equipment as specified by the Contracting Authority. Where such equipment is required, the installation shall be able to be completed during the manufacture of the Vehicle by the Supplier or another party in agreement with the Contracting Authority. |
| --- | --- |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Communications & ICT section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 25. 	Radio Installation

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | All radio installation, communications and wiring must be compliant with agreed National Fire Standard (Airwave/ESN). |
| --- | --- |
|  | All installers must have appropriate certification and competence to undertake the work. |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Radio Installation section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 26. 	CCTV

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the CCTV section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 27. 	Vehicle Control Systems

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | The Contracting Authority may wish to stipulate a Public Address system to support operational requirements as determined through risk assessment. |
| --- | --- |
|  | Where specified by the Contracting Authority the operator shall be provided with controls that give variable and finite control of system operation that may be fixed and or remote. |
|  | The Contracting Authority may require devices to monitor and record systems status and performance for both operator and management information. |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Vehicle Control Systems section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 28. 	Emergency Visual and Audible Warning Equipment

The following compliant framework requirements are mandatory and must be met by the Suppliers. Please declare below that you met these compliant framework requirements:

Yes ☐   No ☐

|  | To promote safe progression through traffic, warning other road users and protection of operators at the incident, the vehicle shall be fitted with visual and audible emergency warning systems in accordance with the Contracting Authority’s requirements. |
| --- | --- |
|  | Lock and open/shut status shall be audibly and visually indicated at the point of operation and be designed as to fail and designed to fail safe i.e., unlocked. |

Please provide details in the box below if you do not comply with any of the compliant framework requirements including the reference and the rationale for non-compliance:

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Emergency Visual and Audible Warning Equipment section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

## 29. 	Additional Requirements

The following table shows the Contracting Authority’s Requirements. The Supplier must confirm if they comply with each requirement and provide a detailed response to each requirement to show how the requirement will be met.

|  | Requirement | Compliant (Yes / No) | Supplier’s Response |
| --- | --- | --- | --- |

The following table shows the questions that relate to the Additional Requirements section:

| Ref | Question | Weighting |
| --- | --- | --- |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |
|  | [add in question topic / questions / sub criteria] | [add in pass / fail or scored and any weighting] |
|  | Response: Suppliers may complete this box and/or refer to clearly cross reference attached documents |  |

# Section Four – Equipment List

| No. | Description of Equipment | Qty | Who provides the equipment? | Weight (if known) | Additional Information |
| --- | --- | --- | --- | --- | --- |